Design a digital circuit that counts the number of 1’s in a 7-bit shift register. You should use T flip-flops for your circuit. Your circuit should display the value on a seven segment display. (Note that you do not need to design the seven segment display decoder, you can use the following decoder.)

Design a four-bit universal shift register that operates the following modes in the given order. Draw the circuit.  Show the function table.

Register operations:	0	→	Set to zero

1	→	Shift right

2	→	No change

3	→	Load parallel data

4	→	Shift left

5	→	No change

6	→	Complement

7	→	Set to 1.